MEDIA VISION OS/2 DRIVERS

==============================

May 27, 1993

This file contains important information for OS/2 2.1 users that have

installed Media Vision's OS/2 2.1 MMPM drivers.  Along with this file

you should have received a SCSI driver and a Presentation Manager mixer

program.

Audio driver

The audio driver for the Multimedia Presentation Manager subsystem can

be easily installed from the OS/2 distribution disks.

The MMPM/2 audio driver is named MVPRODD.SYS.  MVPRODD now supports many of

the same command line switches that  the DOS device driver MVSOUND.SYS uses.

Here is a list:

/S:X,XXX,X,X     SoundBlaster { enable,base addr,DMA,IRQ }

Note: Sound Blaster DMA channel must be 1 on this

driver release.

/N:<name>        allows naming of driver /N:PAS161$ (do not change)

/T:X             T:1 = use PAS oscillator for OPL-3 (default /T:0)

/D:X             sets the DMA channel

/Q:X             sets the IRQ channel

/I:X             also can be used to set the IRQ channel

/B:XXX           HEX Base board I/O location (DEFAULT: /B:388)

/W:X             /W:1 enables warm boot reset (DEFAULT /W:0)

/M:X,XXX,X       MPU {enable,base addr,IRQ}

/F:X             FM Synth disable switch; /F:1 enabled by default

/J:X             /J:1 cause Joystick to be enabled, J:0 is default

I/O relocation.

The PAS Plus and PAS 16 have always been able to

reside at addresses other than the default port addresses: 0x388-0x38b.

This is useful when there are hardware conflicts with another board

that wants to use the same I/O space or when the user wishes to use

multiple PAS cards.   If you relocate the I/O addresses, however, be

forewarned that DOS applications and tools usually will not find the

card.

Warm Boot Reset.

The Warm Boot Reset feature is useful to clear the tones generated by

an Adlib compatible game that has been aborted with CTRL-ALT-DEL.

This feature has been traditionally enabled in the past.  The PAS

detects a warm boot reset (CTRL-ALT-DEL) by decoding writes to the DMA

refresh timer.  Some new motherboards (system planars) and reportedly

BIOS' from Quadtel use that same timer for something else, causing the

PAS to constantly reset.  It's therefore safer to have this feature

default to off.  Try adding the switch and if the card still produces

sound, it's not a problem for your system.

Sound Blaster Initialization.  The PAS 16 has dedicated Sound Blaster

circuitry which operates independently of the high quality digital

audio section.  That's why the PAS 16 requires two DMA's and two IRQ's.

The /S switch will enable the SoundBlaster circuitry allowing game

compatibility in a DOS box and also allowing the snblst2.drv and

sbwave.drv drivers to be used in WINOS2.

/S:1,220,1,5    is the correct setting if using the factory defaults

Notes on MVSOUND.SYS in OS/2 DOS sessions:

The DOS device driver has three main purposes:

1) Initialize PAS 16 hardware, setting mixer volumes

2) Initialize the Sound Blaster circuitry

3) Provide volume control services to DOS apps.

Since the OS/2 driver, MVPRODD.SYS, now provides items 1 and 2 above,

it may not be necessary to load MVSOUND.SYS in your DOS & WinOS2

session.  However, if you are using a DOS application that requires

MVSOUND.SYS (e.g. PLAYFILE, Trak Blaster Pro or the PAS utility)

you will have to load it.  '

Some games, Wolfenstein 3D for example,  require the FM synthesizer

chip (the OPL-3) to be in Adlib-compatibility mode.  The MVPRODD.SYS

sets the synthesizer chip for enhanced OPL-3 mode.  When starting a

DOS session, the synth chip will be in the wrong mode and thus

the game won't recognize the hardware.  In cases like this it is

necessary to load MVSOUND.SYS in the DOS box.  In the future we will

handle the OPL-3 mode conversion automatically.

The file PROAS.EXE, which is available on the Media Vision BBS and

CompuServe's MULTIVEN forum (GO MEDIAVISION), is a program that

performs the same functions as MVSOUND.SYS but can be run from

autoexec.bat and does not stay resident.  It might be useful in

situations where DOS memory is at a premium.

MPU-401 emulation.  This requires a PAS 16 with a Rev 'D' MV101 chip.

To check your PAS 16 chip version number, run DEBUG and type

'i FF88'  (this assumes card I/O addressing has not been relocated)

If the value is 4, you have a Rev D chip and you can enjoy MPU-401

(UART mode) emulation in DOS session.   If you have an older Rev

chip, we hope to emulate MPU-401 hardware with a VDD in the future.

For most systems the following settings will work:

/M:1,330,15

--------------------------------------------------------------------------------

FREQUENTLY ASKED QUESTIONS

--------------------------------------------------------------------------------

Q: How can I control the mixer functions of the PAS card?

A: Media Vision has developed an application, PROMIX2.EXE, that provides

complete access to the PAS mixing functions.  Promix2 is available

on CompuServe and on the Media Vision BBS.  Updates to drivers, new

OS/2 programs and third party software will also be available there.

The Promix2 application requires OS2MIXER.DLL to be in the LIBPATH.

The MMOS2\DLL directory is in the libpath.

copy OS2MIXER.DLL to c:\MMOS2\DLL

Copy the Promix2.exe application to the MMOS2 directory.

copy PROMIX2.EXE to c:\MMOS2

Copy the Promix2 help file to the MMOS2 directory.

copy PROMIX2.HLP to c:\MMOS2

You may wish to add an icon to your desktop for launching the Promix2

application.  The easiest way to do this is by dragging the Program

template from your templates folder to the desktop.  By changing the

settings on the Program and General pages, you'll be able to launch

Promix2.

Promix2's help facility is activated via the F1 keystroke.

Q:  What Media Vision Products are supported?

A:  Audio support

--------------

(***)

Pro AudioSpectrum Plus (see MMPM2.INI file change required below)

Pro AudioSpectrum 16

Pro Audio Studio 16 (++)

Pro 16 Multimedia Upgrade Kit

Fusion CD

Fusion CD 16

Fusion CD 16E

Fusion Double CD

CDPC  (**) (sound only no CD support)

CDPC XL (**)

(**)  The CDPC and CDPC-XL front panel volume and mute controls are not

yet supported.

(***) Audio support for the original Pro AudioSpectrum (8-bit ISA bus

slot interface) has not been tested for this release. OS/2

MMPM support will be available in a future release.

(++) Pro AudioStudio 16's selectable input gain feature is not supported

on this release.

Q: If I have trouble, what should I do?

A: Media Vision's Technical Support department telephone number is

(800) 638-2807.  Media Vision also has a European office that

offers technical assistance.  They are located in Munich.  Check

directory assistance for Media Vision GMBH.

Media Vision, Inc        Main:         (800) 348-7116 or (510) 770-8600

3815 LaurelView Ct.      Sales:        (800) 845-5870

Fremont, CA 94539        Parts:        (800) 356-7886

Tech Support: (800) 638-2807 or (510) 770-9905

FAX:          (510) 770-8648 or (510) 770-9592

BBS: (510) 770-0968 2400 (8 bits, no parity, 1 stop bit)

(510) 770-1661 2400 (8 bits, no parity, 1 stop bit)

(510) 770-0527 9600 (8 bits, no parity, 1 stop bit)

Developer Assistance Program: (800) 472-6147

CIS: GO MEDIAVISION or send CIS mail to 75300,2772

Internet: Send mail to 75300,2772@compuserve.com

Q: Which .WAV files can I play.

A: You should be able to play any PCM (uncompressed) .WAV file

with sampling rates between 4 and 48 KHz, mono or stereo, and

8 or 16 bits.

Q: How can I change my card's DMA and IRQ settings?

A: The PAS Plus, PAS 16 and CDPC all have software configuration settings.

The 'device=' line in config.sys can be used to specify different IRQ

and DMA settings.

DEVICE=c:\os2\mvprodd.sys /I:11 /D:3

The /I parameter is used to set the IRQ.  The /D parameter is used to

set the DMA.

Valid IRQ settings are: 2,3,4,5,6,7,10,11,12,14,15

Valid DMA settings are: 1,2,3,5,6,7

DMA channels 5,6,7 use 16-bit data transfers and are preferable choices.

Q: Will I be able to run my DOS games and applications?

A: YES!  The IBM team has done a superlative job at providing support

for sound hardware using auto-init DMA.  You can actually run

TrakBlaster Pro in a DOS session!  You will need to load MVSOUND.SYS

for the DOS session.  Make sure that your DOS IRQ and DMA settings

match those used by the OS/2 device driver.

If you have trouble with Sound Blaster compatible games, make sure

you add the /S switch (details above).  Also, some games (Wolfenstein

3D, for example) check a DOS enviroment variable for IRQ and port address

information.

Add the line

set blaster=a220 d1 i5 T3

to your autoexec.bat file.  This assumes you are using the factory

default Sound Blaster IRQ setting of 5.

Q: Will I be able to run the Media Vision Windows drivers in WIN/OS2?

A: Yes.  Consider using the Sound Blaster section of the card for

your WinOS2 multimedia audio driver.  With such a configuration, you

will be able to use Windows Multimedia utilities and OS/2 multimedia

utilities at the same time.  When using the FM, MIDI or PAS digital audio

features in WinOS2, you will not be able to use these features in OS/2

Q: Can I run the Sound Blaster OS/2 drivers that came with OS/2 on the

PAS 16?

A: No.  The Sound Blaster drivers, which were written by Creative Labs

check for Media Vision hardware and fail if they detect a PAS 16,

ThunderBoard or Thunder and Lightning card.

Q: Why do volumes change between WinOS2 and OS/2?

A: At present mixer volumes are not globally consistant.  When starting a

WinOS2 session, expect the Windows drivers to reset mixer settings.

Q: I have a Pro AudioSpectrum Plus and I can't record or play audio

under OS/2 2.1.  What's wrong?

A: In the file MMPM2.INI found in your MMOS2 directory, you'll find a

header like this:

[ibmwavepas1601]

Under this heading you will find the PARMSTRING parameter.  Change

the string BPS=16 to BPS=8 and digital audio will work.

Q: Can I use these drivers with OS/2 2.0?

A: OS/2 2.1 is required for audio support.  The SCSI driver will work on

both OS/2 2.0 and OS/2 2.1.

Q:  Can I play CD audio?

A:  The SCSI driver will support CD (Redbook) audio playback when used in

conjunction with IBM's MMPM/2 system software.  It will also support

most SCSI hard disks.

Q:  What CD-ROM drives are supported?

A:  CD-ROM Drive Support List

-------------------------

Hitachi   CDR-1650S, CDR-1750S, CDR-3650, CDR-3750

IBM       CD-ROM I, CD-ROM II

NEC       CDR-25J,CDR-36, CDR-37, CDR-72, CDR-73, CDR-74, CDR-82, CDR-83,

CDR-84

Panasonic CR-501, LK-MC501S

Sony      CDU-541, CDU-561, CDU-6111, CDU-6211, CDU-7211

Texel     DM-3021, DM-3024, DM-5021, DM-5024

Toshiba   XM-3201, XM-3301, XM-3401

Support is provided for both data and audio on all models.

If you have a "CDPC", "Fusion CD 16" or "Media Vision Multimedia

Upgrade Kit " you should have a CD-ROM drive with a Sony interface.

The Sony interface is not yet supported.  See the release notes above.

Q: Is Kodak Photo CD supported?

A: Single session, Kodak Photo-CD support is provided for those drives which

support this standard.

Q: How fast is the SCSI interface under OS/2

A: We have measured hard file (hard disk) reads at 504 Kb/sec and CD-ROM

(NEC 83j) reads at 238 Kb/sec.