# Common Problems

## "Cannot open file <-undef>" during compilation

The DOS compiler driver and compiler tools are not intercompatible with the Win32 driver and tools. The most frequent symptom of a clash is the above message from the compiler driver. If you get this error you should immediately check that your tools are all DOS or preferably all Win32.

## "Stack Fault at eip=d7091 … Call frame traceback EIPs …" during compilation

If you get an error like this it's almost certainly because you're using the DOS compilers and they have exhausted available stack memory during a heavy optimisation. Either get a newer build of the compiler (builds before June 1996 were built with smaller stacks) or switch to the Win32 tools, which don't suffer this problem.

## "Instruction patch out of range" during linking

On the Saturn there is a very rare compiler bug which causes this. Contact Sega for help.

On the Playstation this might happen if too many items are being assigned to the .sdata and .sbss sections by the -G flag (default value 8). There is a practical limit of 32K on the data which can be stored in these sections; references to them are evaluated by a 16-bit offset from the GP register. If the patches in question refer to these sections, recompile your files with a smaller -G value and try again.

On the Nintendo 64 the GP register and the .sdata and .sbss sections aren't supposed to be used. You must compile with -G0 for the Nintendo 64.

If the patch is referring to another section, perhaps you have ORGed the program to one address but OBJed certain groups to other locations, and you have forgotten to correctly allocate both sections to a group, so one has an OBJ address and the other doesn't.

## "Can't open library file" when doing psylib /l

This is probably because the library file is read-only. Versions of psylib before 2.07 and psylib2 before 2.02 had an error which made them try to open files for read-write even if they were intending only a read operation.

## "__main multiply defined" on Saturn

You are linking with libgcc.lib specified before libsn.lib in your list of libraries. Make sure libsn.lib appears first.

## Source filename not stored with correct path on Saturn

This happens on the DOS tools and is a compiler bug outside our control. The 32-bit compilers have this fixed.

## "Can't find symbol '_link' (or '_unlink')" on Saturn

And with reason. 'link' and 'unlink' are standard C library members which create and destroy files on the file system. Cygnus has left them as unresolvable precisely because you shouldn't be using them. If you get this error you are probably converting a PC game which had file-handling routines, which you will have to recode anyway.